Required Hardware Additions

1. Push Button (Panic / SOS)

Purpose:

Manual emergency trigger Sends immediate SOS alert with location

Components: Tactile push button , Internal pull-up (no resistor needed)

Connection:

One leg → GPIO 4

Other leg → GND

2. Buzzer (Alert System)

Purpose:

Audible alert when: Child leaves geo-fence

SOS triggered

Components: Active buzzer

Connection:

Positive → GPIO 5

Negative → GND

3. GSM Module (SIM800L / SIM900)

Purpose:

Send SMS alerts and receive commands (TRACK)

Components:

SIM800L (preferred)

SIM card (SMS enabled)

External power supply

Connection:

TX → ESP32 RX (GPIO 16)

RX → ESP32 TX (GPIO 17)

GND → GND

VCC → 3.7V ( NOT 5V)

4. LED Indicator

Purpose:

System status and alert indication

Components:

LED + 220Ω resistor

| Component | Required | Purpose |
| --- | --- | --- |
| Push Button | Yes | SOS trigger |
| Buzzer | Yes | Alert sound |
| GSM Module | Yes | SMS + commands |
| LED | Optional | Status |

Important Notes

- GSM requires stable power (~2A peak)

- All modules must share common ground

- Keep GSM antenna away from ESP32

- Use regulator / battery for stability

UPDATED CODE: C++

#include <Arduino.h>

// Pins

#define BUTTON_PIN 4

#define BUZZER 5

// Geo-fence (school area)

float schoolLat = 12.970972;

float schoolLng = 79.164083;

float radius = 0.001;

// Simulated GPS

float currentLat = 12.970972;

float currentLng = 79.164083;

unsigned long lastAlertTime = 0;

// Button state

bool lastButtonState = HIGH;

// Buzzer

void triggerBuzzer() {

digitalWrite(BUZZER, HIGH);

delay(1000);

digitalWrite(BUZZER, LOW);

}

// Simulated SMS

void sendSMS(String message) {

Serial.println("SMS SENT:");

Serial.println(message);

Serial.println("----------------------");

}

// Alert

void sendAlert() {

String msg = "ALERT! Child left safe zone\n";

msg += "https://maps.google.com/?q=";

msg += String(currentLat, 6) + "," + String(currentLng, 6);

sendSMS(msg);

triggerBuzzer();

}

// SOS

void sendSOS() {

String msg = "SOS! Emergency!\n";

msg += "https://maps.google.com/?q=";

msg += String(currentLat, 6) + "," + String(currentLng, 6);

sendSMS(msg);

triggerBuzzer();

}

// Button handling

void handleButton() {

bool currentState = digitalRead(BUTTON_PIN);

if (lastButtonState == HIGH && currentState == LOW) {

Serial.println("SOS BUTTON PRESSED");

sendSOS();

delay(300);

}

lastButtonState = currentState;

}

void setup() {

Serial.begin(115200);

pinMode(BUTTON_PIN, INPUT_PULLUP);

pinMode(BUZZER, OUTPUT);

Serial.println("SMART CHILD SAFETY SYSTEM STARTED");

}

void loop() {

handleButton();

// ---------- TEST CASE 1: NORMAL ----------

// Keep these values → child inside safe zone

currentLat = 12.970972;

currentLng = 79.164083;

// ---------- TEST CASE 2: GEOFENCE VIOLATION ----------

// Uncomment below lines to simulate child outside safe zone

/*

currentLat = 12.975000;

currentLng = 79.170000;

*/

// ---------- TEST CASE 3: SOS ----------

// Click the push button in simulation to trigger SOS

int hour = 10;

bool schoolTime = (hour >= 8 && hour < 14);

float distance = sqrt(

pow(currentLat - schoolLat, 2) +

pow(currentLng - schoolLng, 2)

);

bool outside = (distance > radius);

if (schoolTime && outside) {

if (millis() - lastAlertTime > 5000) {

Serial.println("GEOFENCE VIOLATION");

sendAlert();

lastAlertTime = millis();

}

} else {

Serial.println("Child is inside safe zone");

}

Serial.println("---------------------------");

delay(2000);

}

“Initially, the system only sent GPS data.
Now we added decision-making using geo-fencing and time logic.
The system automatically detects unsafe conditions and sends alerts.
It also includes a manual SOS feature and on-demand tracking via SMS.”

Show these 3 cases:

1️⃣ Normal

✔ “Inside safe zone”

2️⃣ SOS

✔ Button press → SOS message

3️⃣ Violation

✔ Change location → alert

“We validated the system in simulation by testing normal conditions, emergency SOS, and geo-fence violations. The system correctly detects unsafe conditions and triggers alerts.”

We used edge detection with debounce delay to ensure reliable SOS triggering.